Stuff for Tracey – documentation for research on the VS1053 codec line-level IO

RaspberryPi

| Links | General code for any microcontroller to use the VS1053 to recording and playback, written in C, here – stopped pursuing because Arduino was showing more progress, so change of plans: have everything work on Arduino, then port it to the pi. |
| --- | --- |
| Resources | My github has the recording capabilities for a USB input mic and documentation written up upon previous request, here. |

Arduino

| Links | No code provided, but describes how to write the VLSI ADMixer plugin to the SD card to decode the audio input and eventually decode and playback, here. |
| --- | --- |
|  | To see a forum on using the codec as a music player and voice amplifier, here. |
|  | For other VLSI solutions plugins for different encoding formats and techniques for manipulating the audio data with individual documents on implementation, here. |
|  | A project that uses the MAX4466 Electret Mic to record 9 hours activated with a button press, here. |
|  | Specific VS1053 audio input/output, addresses, modes, and registers, here. |
| Attachments |  |
|  | “ |
| Still Working | Have played with adafruit blinka and circuit python bundles, seems like a dead end so far. |

Files (more details in sketches)

sineTest – checks pins are connected properly by sending a tone to LOUT/ROUT without SD card

Adafruit_VS1053.h/.cpp - contains the edited libraries my .ino files ran because I changed some data member from private to protected for testing sineTest, but the one from the internet, here, should work fine with other sketches.

Pin Connections

(VS1053 -> ARDUINO)

XDCS -> D8

SDCS -> D4

CS -> D10

RST -> D9

SCLK -> D13

MOSI -> D12

MISO -> D11

GND -> GND

VCC -> 5V

DREQ -> D3

(MAX4466 -> VS1053)

VCC -> 5V

GND -> AGND

OUT -> LINE2

Other people with mic connection issues, here.

(VS1053 -> HEADPHONE SOCKET)

AGND -> MIDDLE PIN

LOUT/ROUT -> OUTER PINS

To Do

-use recorgOgg to upload ADMIXER plug in (plugins found here) to encode in mp3 wav format (?)

-this strat forum to use this setup as a real-time mic amplifier

Notes

MAX 4466 datasheet says to bypass the power supply with a 0.1µF capacitor to ground

As of 07/11/19

- Customizing the libraries (SD and Adafruit_CS1053) as I go

- Everything in the admixer-plugin-record.ino because eventually will have that plugin decode input as its encoding it.

Done:

- Adafruit_VS1053_FilePlayer object, musicPlayer is flawless

- The sketch has a sinTest at the beginning to make sure communication with pins is working

- The SD card is readable and prints the directory to prove it

In progress (aka my current headache):

- Play an MP3 file on the SD card code (Adafruit_VS1053_Library > examples > simple_player.ino)

  - playFullFile()

    - calls startPlayingFile()

  - startPlayingFile()

    - track is identified

    - verified to be a MP3 file

    - calls feedBuffer()

  - feedBuffer()

    - does stuff with interrupts

    - changes control flag  boolean, feedBufferLock

    - calls feedBuffer_noLock()

  - feedBuffer_noLock()

    - calls playData()

  - playData()

    - reads single value mp3buffer and writes to the register, which should play it, but doesn’t…

For this output:

This is what the playFullFile() looks like:

To see each function and call and what not, it can be found in mt edited “Adafruit_VS1053” files

To Do:

- Activate and use  plugin to stream audio in real time, in thru MIC out through LOUT/ROUT

Done/Doing:

Read from the SD card successfully with “read-from-SD”

- Run read-from-SD

- Used VLC player to double check